ASRockRack Technical Document (May/2016)

Q: How to update E3C224D4I-14S on-board LSI 2308 to IR mode?

Update flow:

- Unzip “E3C224D4I-14S_IR_L_20_14.01.00.01.legacy” to bootable USB Drive.

- BIOS setting:  Advanced ->Storage Configuration ->LSI Controller : Enable

->Bootable LSI Controller : Yes

- Boot to DOS environment.

- Execute readaddr.bat (Program will save SAS address to info.txt in USB drive)

Example:

- Execute start_IT.bat to update LSI2308 FW in folder “LSI2308_P17_IT”, process will start and please wait 4 minutes.

- Restart and press Ctrl+C into LSI RAID ROM and you will see LSI2308 has been updated to IT mode.

===================================================================

Note:

- If SAS address have been changed please type command (in DOS)

“sas2flsh.exe -o -sasadd XXXXXXXXXXXXXXXX” Your SAS address

- Restart and check SAS address

- Finish