ASRockRack Technical Document (Aug/2020)

Q: How to re-write the LAN MAC address?

| Preparation | : |
| --- | --- |

- USB drive

- “Eeupdate” folder and copy it into USB drive

- Boot into Built-in EFI Shell environment

Steps:

- Check all the LAN MAC address on the mainboard.

Command:		eeupdate64.efi /all /mac_dump

Sample:

- Write the LAN(s) MAC address.

LAN MAC address: refer to the white sticker on the mainboard. (If there only one sticker, that is LAN1 MAC address (HEX); plus 1 will be LAN2 MAC address and so on)

Command:		eeupdate64.efi /nic={LAN#} /mac={LAN# MAC address}

Sample:

- Reboot the system after the LAN# MAC address update process finished.

----- END -----

Sampel of MAC sticker: