### ASRockRack Technical Document (Jan/2019)

Q: How to flash BMC FW in DOS?

| Preparation | : |
| --- | --- |

- A bootable USB drive

(P.S. you can build a boot USB drive by rufus https://rufus.akeo.ie/ )

- Update BMC Batch files folder

Step:

- Copy your BMC image to “Socflash” folder

- Use a bootable USB drive boot to DOS.

- Execute command “Socflash.exe {update image name} {backup image name}” to flash BMC firmware.

Ex: Socflash.exe EPC612D8L1.10 backup.ima

Flash process will take about 5 minutes, please wait.

- Check BMC version in BIOS.

========================================================

Flash BMC firmware completed!

*If MAC address of IPMI and NCSI became 00-00-00-00-00-00, please use BMC Mac backup Tool in BIOS-> Server Mgmt-> Restore BMC MAC from backup to restore MAC address.