How to use “S_BMCMAC” to update MAC address

- Prepare a bootable USB disk and “S_BMCMAC.rar”

- Decompress “S_BMCMAC.rar” and put it into USB.

- Choose usb to boot up.

- Find S_BMCMAC.exe and enter below command:

- If the MB have only dedicate Lan:

S_BMCMAC.exe 1={IPMI MAC address}

- If the MB have dedicate Lan and share NIC:

S_BMCMAC.exe 1={IPMI MAC address} 2={NCSI MAC address}

- After you finish update MAC address, please re-boot the system and check if MAC address has been changed successfully or not.