ASRockRack Technical Document (August/2022)

Q: How to update BMC under EFI Shell?

Preparation:

The BMC image file and BMC update tool should be at hand, here’s the update tool:

Step:

- Decompress “uflash_v12207.zip” under a bootable USB drive.

- Put the BMC image file you wish to update to in the same directory.

- Boot up your system with UEFI: Built-in EFI shell.

(Press F11 while POSTing to enter boot menu and choose it)

- Execute: socflash.efi if= *image_file_name*

- Wait until the update is finished. (It will show a message like “Update Flash Chip OK”)

- Reboot and check the BMC version in the BIOS page.

*If BMC version doesn’t show up, please disconnect AC power from the system for at least 20 seconds, then plug AC back on and check again.

-DONE-