BIOS programmer user guide

ASRockRack Technical Document (May/2019)

Preparation:

- Operating system: WIN7(32bit 64bit)/WIN8。

- Driver: install by CD in programmer box

Steps:

- Insert BIOS programmer to your computer

- Use the CD in BIOS programmer box to install driver

- Follow the instruction to install driver

- After install complete, please open CH2013

- Check the link status at the bottom right of the window, then choose device type as “SPI flash”

- Choose manufacturer randomly

- Press detect to check if it can get the right ROM chip type (Ex: We send you the chip made by MXIC )

- Click open to select the BIOS image you want to update

- Click Auto to flash ROM chip

- During process

- It will show message at the bottom right after flash complete