BMC Update locally under Linux SOP

OS:

CentOS 7.5 , Ubuntu 14.04 and Ubuntu 18.04

Tool:

Step:

- Put the BMC FW image and the Yafuflash tool in the same folder.

- Go to the folder in command line.

- Enter the following command in command line:

Use root permission

- chmod +x Yafuflash

- chmod +x <BMC.ima>

- rmmod ipmi_devintf

- rmmod ipmi_si

- rmmod ipmi_ssif

- rmmod ipmi_msghandler

- ./Yafuflash -cd -fb -full <BMC.ima>

- Enter the following command in command line when update is complete:

- modprobe ipmi_devintf

- modprobe ipmi_si

- modprobe ipmi_ssif

- modprobe ipmi_msghandler