Update Tool : Yafuflash

BMC Chip : 2500 and 2400

Update Method : Local

OS : Windows server 2016 and Windows server 2012 R2

Update step :

- Put the BMC FW image and the Yafuflash tool in the same folder.

- Go to the folder in command line.

- Enter the following command in command line: Yafuflash.exe -cd -fb -full <BMC.ima>

BMC Chip : 2500 , 2400 and 2300

Update Method : Remote

OS : Windows server 2016 and Windows server 2012 R2

Update step :

- Put the BMC FW image and the Yafuflash tool in the same folder.

- Go to the folder in command line.

- Enter the following command in command line:

Yafuflash.exe -nw -fb -full -ip <BMC ip> -u <user> -p <password> <BMC.ima>