| Date | Version | Description |
| --- | --- | --- |
| 2018-10-12 | v01 | Preliminary draft |

Note: Because it can’t update BMC firmware via Yafuflash v5.7.0.1.0 under Ubuntu (Over LAN) please note the Yafuflash version. The version in red words indicates it can’t update successfully.

Note: 5* indicates the version of Yafuflash is v5.7.0.1.0.

Note: Click the green word and press the Ctrl can link to more information.

AST2500

|  | OS/Tool | Yafuflash | Curl tool (with python2.7.X) | Socflash |
| --- | --- | --- | --- | --- |
| Windows | Windows server 2016 | Remote/Local | Remote | X |
|  | Windows server 2012 R2 | Remote/Local | Remote | X |
| Linux | CentOS 7.5 | Remote/Local | Remote | X |
|  | Ubuntu 14.04 | Remote(5*/4.89)/Local | Remote | X |
|  | Ubuntu 18.04 | Remote(5*/4.89/2.32)/Local | Remote | X |
| Dos |  | X | X | Local |

AST2400

|  | OS/Tool | Yafuflash | Curl tool | U_BMC |
| --- | --- | --- | --- | --- |
| Windows | Windows server 2016 | Remote/Local | X | X |
|  | Windows server 2012 R2 | Remote/Local | X | X |
| Linux | CentOS 7.5 | Remote/Local | X | X |
|  | Ubuntu 14.04 | Remote(5*/4.89)/Local | X | X |
|  | Ubuntu 18.04 | Remote(5*/4.89/2.32)/Local | X | X |
| Dos |  | X | X | Local |

AST2300

|  | OS/Tool | Yafuflash | Curl tool | U_BMC |
| --- | --- | --- | --- | --- |
| Windows | Windows server 2016 | Remote/Local | X | X |
|  | Windows server 2012 R2 | Remote/Local | X | X |
| Linux | CentOS 7.5 | Remote/Local | X | X |
|  | Ubuntu 14.04 | Remote(5*/4.89)/Local | X | X |
|  | Ubuntu 18.04 | Remote(5*/4.89/2.32)/Local | X | X |
| Dos |  | X | X | Local |

Update Tool : Yafuflash

BMC Chip : 2500 and 2400

Update Method : Local

OS : Windows server 2016 and Windows server 2012 R2

Tool : \\ar-file-01\BMC\BMC\Utility\BMC_Update\Yafuflash\Yafuflash_v5.7.0.1.0\Win64

Update step :

- Put the BMC FW image and the Yafuflash tool in the same folder.

- Go to the folder in command line.

- Enter the following command in command line: Yafuflash.exe -cd -fb -full <BMC.ima>

BMC Chip : 2500 and 2400

Update Method : Local

OS : CentOS 7.5 , Ubuntu 14.04 and Ubuntu 18.04

Tool : \\ar-file-01\BMC\BMC\Utility\BMC_Update\Yafuflash\Yafuflash_v5.7.0.1.0\Linux_x86_64

Update step :

- Put the BMC FW image and the Yafuflash tool in the same folder.

- Go to the folder in command line.

- Enter the following command in command line:

Use root permission

- chmod +x Yafuflash

- chmod +x <BMC.ima>

- rmmod ipmi_devintf

- rmmod ipmi_si

- rmmod ipmi_ssif

- rmmod ipmi_msghandler

- ./Yafuflash -cd -fb -full <BMC.ima>

- Enter the following command in command line when update is complete:

- modprobe ipmi_devintf

- modprobe ipmi_si

- modprobe ipmi_ssif

- modprobe ipmi_msghandler

BMC Chip : 2500 , 2400 and 2300

Update Method : Remote

OS : Windows server 2016 and Windows server 2012 R2

Tool : \\ar-file-01\BMC\BMC\Utility\BMC_Update\Yafuflash\Yafuflash_v5.7.0.1.0\Win64

Update step :

- Put the BMC FW image and the Yafuflash tool in the same folder.

- Go to the folder in command line.

- Enter the following command in command line:

Yafuflash.exe -nw -fb -full -ip <BMC ip> -u <user> -p <password> <BMC.ima>

BMC Chip : 2500 , 2400 and 2300

Update Method : Remote

OS : CentOS 7.5 and Ubuntu 14.04

Tool for CentOS 7.5: \\ar-file-01\BMC\BMC\Utility\BMC_Update\Yafuflash\Yafuflash_v5.7.0.1.0\Linux_x86_64

Tool for Ubuntu 14.04: \\ar-file-01\BMC\BMC\Utility\BMC_Update\Yafuflash\Yafuflash_v4.89.0\Linux_x86_64

Update step :

- Put the BMC FW image and the Yafuflash tool in the same folder.

- Go to the folder in command line.

- Enter the following command in command line:

Use root permission

- chmod +x Yafuflash

- chmod +x <BMC.ima>

- ./Yafuflash -nw -fb -full -ip <BMC ip> -u <user> -p <password> <BMC.ima>

BMC Chip : 2400 and 2300

Update Method : Remote

OS : Ubuntu 18.04

Tool : \\ar-file-01\BMC\BMC\Utility\BMC_Update\Yafuflash\Yafuflash_v2.32\Linux_x86_64

Update step :

- Put the BMC FW image and the Yafuflash tool in the same folder.

- Go to the folder in command line.

- Enter the following command in command line:

Use root permission

- chmod +x Yafuflash

- chmod +x <BMC.ima>

- ./Yafuflash -nw -full -vyes -ip <BMC ip> -u <user> -p <password> <BMC.ima>

BMC Chip : 2300

Update Method : Local

OS : Windows server 2016 and Windows server 2012 R2

Tool : \\ar-file-01\BMC\BMC\Utility\BMC_Update\Yafuflash\Yafuflash_v5.7.0.1.0\Win64_for_2300

Update step :

- Put the BMC FW image and the Yafuflash tool and ipmiutil tool in the same folder.

- Go to the folder in command line.

- Enter the following command in command line:

- ipmiutil cmd 00 20 c8 cb 01

- Yafuflash.exe -cd -fb -full <BMC.ima>

BMC Chip : 2300

Update Method : Local

OS : CentOS 7.5

Tool : \\ar-file-01\BMC\BMC\Utility\BMC_Update\Yafuflash\Yafuflash_v5.7.0.1.0\Linux_x86_64

Update step :

- Put the BMC FW image and the Yafuflash tool in the same folder.

- Go to the folder in command line.

- Enter the following command in command line:

Use root permission

- yum install ipmitool

- ipmitool raw 0x32 0xcb 0x01

- chmod +x Yafuflash

- chmod +x <BMC.ima>

- rmmod ipmi_devintf

- rmmod ipmi_si

- rmmod ipmi_ssif

- rmmod ipmi_msghandler

- ./Yafuflash -cd -fb -full <BMC.ima>

- Enter the following command in command line when update is complete:

- modprobe ipmi_devintf

- modprobe ipmi_si

- modprobe ipmi_ssif

- modprobe ipmi_msghandler

BMC Chip : 2300

Update Method : Local

OS : Ubuntu 14.04 and Ubuntu 18.04

Tool : \\ar-file-01\BMC\BMC\Utility\BMC_Update\Yafuflash\Yafuflash_v5.7.0.1.0\Linux_x86_64

Update step :

- Put the BMC FW image and the Yafuflash tool in the same folder.

- Go to the folder in command line.

- Enter the following command in command line:

Use root permission

- apt-get install ipmitool

- ipmitool raw 0x32 0xcb 0x01

- chmod +x Yafuflash

- chmod +x <BMC.ima>

- rmmod ipmi_ssif

- ./Yafuflash -cd -fb -full <BMC.ima>

- Enter the following command in command line when update is complete:

- modprobe ipmi_ssif

Update Tool : Curl Tool

BMC Chip : 2500

Update Method : Remote

OS : Windows server 2016 and Windows server 2012 R2

Tool : \\ar-file-01\BMC\BMC\Utility\BMC_Update\Curl\Win64\Win64_FWUpgradeAgent_v1.0.3

Update step :

- Install python 2.7.x and add python.exe to PATH.

Note: python 2.7.12 \\ar-file-01\FWQT\TestTool\Python. Also you can download from the official website https://www.python.org/downloads .

- Enter the command “Python -V“ in command line to check the Python version is 2.7.x.

- Enter the following command in command line to install python packages:

- pip install argparse

- pip install logging

- pip install Config

- Put the BMC FW image and the FWUpgradeAgent.py in the same folder.

- Go to the folder in command line.

- Enter the following command in command line:

- FWUpgradeAgent.py -IPs <BMC ip> -U <user> -P <password> -File <BMC.ima>

BMC Chip : 2500

Update Method : Remote

OS : CentOS 7.5

Tool : \\ar-file-01\BMC\BMC\Utility\BMC_Update\Curl\Linux64\FWUpgradeAgent_v1.0.3

Update step :

- Put the BMC FW image in the FWUpgradeAgent_v1.0.3 folder.

- Go to the folder in command line.

- Enter the following command in command line:

Use root permission

- python -V

Note: check the Python version. Please use Python version 2.7.x.

- yum install curl

- chmod +x FWUpgradeAgent.py

- chmod +x <BMC.ima>

- ./FWUpgradeAgent.py -IPs <BMC ip> -U <user> -P <password> -File <BMC.ima>

BMC Chip : 2500

Update Method : Remote

OS : Ubuntu 14.04 and Ubuntu 18.04

Tool : \\ar-file-01\BMC\BMC\Utility\BMC_Update\Curl\Linux64\FWUpgradeAgent_v1.0.3

Update step :

- Put the BMC FW image in the FWUpgradeAgent_v1.0.3 folder.

- Go to the folder in command line.

- Enter the following command in command line:

Use root permission

- apt-get install python

Note: Enter the command “Python -V“ in command line to check the Python version. Please use Python version 2.7.x.

- apt-get install curl

- chmod +x FWUpgradeAgent.py

- chmod +x <BMC.ima>

- ./FWUpgradeAgent.py -IPs <BMC ip> -U <user> -P <password> -File <BMC.ima>

Update Tool : Socflash

BMC Chip : 2500

Update Method : Local

OS : DOS

Tool : \\ar-file-01\BMC\BMC\Utility\BMC_Update\Socflash\socflash_v11500

Update step :

- Put the BMC FW image and the Socflash tool in the same folder.

- Go to the folder in command line.

- Enter the following command in command line:

- socflash if=<BMC.ima>

Update Tool : U_BMC

BMC Chip : 2400 and 2300

Update Method : Local

OS : DOS

Tool :  \\ar-file-01\BMC\BMC\Utility\BMC_Update\U_BMC\v0.3

Update step :

- Put the BMC FW image and the Socflash tool in the same folder.

- Go to the folder in command line.

- Enter the following command in command line:

- U_BMC <BMC.ima>