### ASRockRack Technical Document (OCT/2017)

Q: How to flash BIOS locally in Linux?

| Preparation |  |
| --- | --- |

- BIOS Update Tool for Linux

Execute Environment: Linux

Step:

- Before using AfuLnx64, you have to install some kernel source tool by terminal.

- Open terminal as a super user, then install following package.

apt-get install linux-headers-$(uname -r)

apt-get install dpkg-dev

apt-get source linux-image-$(uname -r)

Apt-get install process is like following picture:

- After Unzip AfuLnx64, you can see the file “afulnx_64”.

- Execute command in terminal: chmod 777 afulnx_64

- Put BIOS image file into AfuLnx64 folder.

- Execute  ./afulnx_64 {your BIOS image file name} /p /b /n

Flash process will take about 3 minutes, please wait.

- Reboot, then check BIOS version.