### ASRockRack Technical Document (Oct/2019)

Q: How to flash EC FW in DOS?

| Preparation | : |
| --- | --- |

- A bootable USB drive

(P.S. you can build a boot USB drive by rufus https://rufus.akeo.ie/ )

- Update BMC tool

Step:

- Copy your EC image to “Socflash” folder

- Use a bootable USB drive boot to DOS.

- Execute command “Socflash.exe if={update image name} option=f” to flash BMC firmware.

Ex: socflash.exe if=b4501~1.bin option=f

- Check EC version in BIOS main page