### ASRockRack Technical Document (Apr/2018)

Q: How to modify BIOS default setting?

| Preparation | : |
| --- | --- |

- BIOS default setting tool : Scelnx_64

Step:

- Open terminal under Linux environment

- Decompress Scelnx_64

- Put your BIOS image in Scelnx_64 folder

- Change Scelnx_64 to executable by entering:

chmod 777 Scelnx_64

- Execute: ./SceLnx_64 /o /s {image name}.txt

- Execute: chmod 777 {image name}.txt

- Edit {image name}.txt to change default

P.S

In this case, we change smbios event log from Enable to Disable.

Original setting:

After changed:

- After finish edit {image name}.txt , please Execute:

./ SceLnx_64 /i /s {image name}.txt to import the BIOS setting

P.S You can ignore the warning message

- Reboot and check if BIOS setting has been changed successfully.