### ASRockRack Technical Document (Aug/2015)

Q: How to modify BIOS default setting?

| Preparation | : |
| --- | --- |

- A boot USB drive

(P.S. you can build a boot USB drive by rufus https://rufus.akeo.ie/ )

- SCEDOS folder and copy it into bootable USB.

Step:

- Export:

Please press command “SCEDOS /o /s <output file name>” in DOS environment to dump the file with the whole variable of BIOS SETUP and generate a file in USB drive.

For Example, Onboard VGA default value = Enabled.

It has three options, Disabled, Enabled & Auto, and its current value will have “*” at top. You can modify it by text editor.

- Import:

With the data we modify from previous file, we have change default to “Auto” and current value to “Auto”.

Then boot to DOS and type “SCEDOS /i /s <input file name>”

Check in BIOS setup menu, the value & default value is changed.

=====================================================================