ASRockRack Technical Document

Q: How to update i210 Firmware under EFI Shell?

Preparation:

Here is the i210 image file and update tool.

Update tool:

I210 NVM rev 3.25:

Step:

- Put “lanconf64e.efi” and “I210A2.bin” to a USB drive.

- Boot up your system with UEFI: Built-in EFI shell.

(Press F11 while POSTing to enter boot menu and choose it)

- Execute: lanconf64e.efi

- Select and update both i210.

Ex:

- LANConf Menu > EEPROM/FLASH > NVM Image >

- Select “Update Image” icon:

- Select SPC621D8.bin to update.

- Wait until the update is finished.

- Update i210 FW successfully.

- Check the i210 version, it should be NVM Version Rev. 3.25.

- After flash both i210 FW, please do AC on/off once.

-DONE-