SceEfi SOP

Preparation:

Step:

- Decompress SceEfi.zip and put SceEfi folder into your USB

- Insert the USB then boot up the system

- press F11 to enter boot menu and choose “Built-in EFI Shell”

- Choose your usb directory (in this example , path of usb is fs0)

- Enter into the directory SceEfi

- Input SceEfi64.efi /i /s test.txt

- If the message “script file imported successfully” appear , it means OK (You can ignore the warning message)

- Double check if those options are modified successfully.

BIOSAdvancedCPU configurationCPU C-state support “Disable”

BIOSAdvancedchipset configurationPCIE ASPM support “Disable”

BIOSAdvancedchipset configurationPCH PCIE ASPM support “Disable”

BIOSAdvancedchipset configurationPCH DMI ASPM support “Disable”