TPM2.0 TEST SOP

OS : SERVER 2012 R2 -GPT MODE

MB : D1521D4I

P.S: OS must install with UEFI mode

- Prepare a TPM2.0 module

- Put TPM2.0 on TPM pin After ac off

- After Boot up >BIOS > Advanced > Trusted Computing > Check if it can recognize  TPM20 Device > set TPM State to “Enabled” > press F10 to save and leave。

When log into OS。Check if it can see Trusted Platform Module 2.0 at Security Devices

- Open Server Manager

- Find “Add roles and features” > then Press “Next” * 4

- Click BitLocker Drive Encryption > Add Feature > Next  (P.S Don’t press Install at this page)

- Click Restart the destination server automatically required > Yes > Install >
system will automatically reboot

- BitLocker will install automatically after reboot.

- Find Control Panel > System and Security> BitLocker Drive Encryption

(Please reboot again, if you can’t find BitLocker Drive Encryption) > click TPM Administration (at Bottom left corner)> If TPM Status shows “The TPM is ready for use, with reduced functionality” > choose Clear TPM > Restart

*** If TPM Status shows ”The TPM is ready for use” , then please jump to step n. ***

- Press F12 after reboot

- After boot into OS, it will show TPM is ready > Close.

- Check if TPM Status shows ”The TPM is ready for use”

- Open BitLocker Drive Encryption，then choose C: <Turn on BitLocker>

- Insert a USB Flash first > choose “Save to a file” > put your BitLocker decrypt code into USB。

- When the message <Your recovery key has been saved> shows up, please Press<Next> to proceed.

- Choose <Encrypt used disk space only> then choose <Next>

- Enable <Run BitLocker system check> then choose <Continue>

- Reboot the system.

- After you reboot and login to the OS，you can see BitLocker is working at bottom right corner.

- After the encrypt process done，please reboot the system. Then shut down the system after login to the OS

- Remove TPM module on the mother board then boot up, and check if there will appear recovery key message or not.

- Check if the system can login to the OS normally, after put TPM module back to the mother board.