- How to Flash BIOS FW in DOS

- How to Flash BMC FW in DOS

- Flash BIOS FW in DOS

| Preparation | : |
| --- | --- |

- A boot USB drive

(P.S. you can build a boot USB drive by rufus)

- Update BIOS batch folder

Step:

- Copy BIOS ROM to “AFUDOS2.39_ASR01” folder.

- Use a boot USB drive booting to DOS.

- Execute command “AFUDOS <ROM_NAME.rom> /p /b /n /asr /x” to flash BIOS firmware.

- Check BIOS version in BIOS.

=====================================================

Flash BIOS firmware completed!

- Flash BMC FW in DOS

| Preparation | : |
| --- | --- |

- A boot USB drive

(P.S. you can build a boot USB drive by rufus)

- Update BMC Batch files folder

Step:

- Copy BMC ROM to “FlashBMC_DOS” folder and modify BMC ROM name to ROM.ima

- Use a boot USB drive booting to DOS.

- Execute command “SOCwMAC.exe” to flash BMC firmware.

Flash process will take about 5 minutes, please wait.

- Check BMC version in BIOS.

========================================================

Flash BMC firmware completed!