- Link by stunnel

OS: windows

Stunnel Download link: http://www.stunnel.org/downloads.html

Generate SSL key (optional)

- To generate SSL key, use a linux OS, open terminal and execute:

openssl genrsa -out privkey.pem 2048

- openssl req -new -key privkey.pem -out cert_req.pem

- openssl x509 -req -days 365 -in cert_req.pem -signkey privkey.pem -out sign_cert.pem

- cat privkey.pem sign_cert.pem > server.pem

- Upload the newly created SSL certificate and private key in the BMC via webpage.

Path: IPMI ->Settings->SSL settings->upload SSL certificate

Install stunnel

- If install successful, stunnel icon should appear in windows task bar (down right)

- Right click on the Stunnel icon, and select Edit Configuration

- Add entry for VNC client as given below and save

[VNC]

client = yes

accept = 127.0.0.1:5678

connect = 10.0.124.22:5901

cert = C:\server.pem

- Reload configuration

- Launch VNC client with loopback IP address

- Link by putty.ssh

- Download putty

- Open command prompt and navigate to directory where putty.exe is placed.

- Enable SSH session with BMC by issuing following command in terminal.

putty.exe -ssh -L {Any Port}:{BMC IP}:{VNC Non Secure Port} {BMC username}@{BMC IP} -pw {BMC password}

Ex: putty.exe -ssh -L 1234:localhost:5900 admin@10.0.124.130 -pw admin

Terminal CMD

Putty

- Link by tightvnc

Download link:

https://www.tightvnc.com/

Steps:

- IPMI page=>setting=>media redirection

- Enable VNC non secure connection

- Use tightvnc viwer to link your IPMI IP

- Success