Instructions for installing and using the new

ID TECH USB/RS-232 Drivers

The new ID TECH USB/RS-232 drivers V3.0 support advanced USB electronics in the latest ID TECH products utilizing a USB/RS-232 interface. They also work with all previous generations of ID TECH products using the USB/RS-232 interface. They replace the ID TECH USB/RS-232 drivers V2.0

For purposes of these instructions, older ID TECH USB/RS-232 devices are referred to as Type A devices. Newer ID TECH USB/RS-232 devices are referred to as Type B devices. There are two drivers to install for each reader: The serial converter driver and the serial port driver.

Installing the Drivers

The following steps are specifically for Windows XP operating system. For Windows 98, 2000, or Windows me, the procedure is similar.

1. Create a folder named "ID TECH USB Serial Driver V3.0." Download the driver from the ID TECH website (www.id-tech.net) and unzip it to this folder.

2. Unplug the network cable to prevent it from downloading a different driver from the Windows website.

3. Go to START/CONTROL PANEL/ADD OR REMOVE PROGRAMS.

 If you find "ID TECH USB Serial Driver V3.0," leave it. The correct drivers are already installed. Proceed to the next step.

 If you find either "ID TECH USB Serial Converter Drivers" or "FTDI USB Serial Drivers," remove them. Then proceed to the next step.

 If you find nothing, proceed to the next step.

4. Plug an ID TECH USB/RS-232 device into one of the USB ports. A "Found new hardware wizard" window should appear on the screen. (If it does not appear, please go to "Checking the Installation and Modifying the COM Port Number.")

5. Select "Install from a list or specific location" and click NEXT. The host should display another window: "Please choose your search and installation options"

6. Select "Don't search, I will choose the driver to install" and click NEXT. You will be asked to select the hardware type. Select "Show all devices" and click NEXT.

7. You will be asked to "select the device driver you want to install for this hardware." Select SHOW ALL DEVICES and HAVE DISK and then click NEXT.

8. The INSTALL FROM DISK window will appear. Click BROWSE.

9. The LOCATE FILE window will appear. Find C:\ID TECH USB Serial Driver V3.0. Select FTDIBUS.INF and click OPEN.

10. Click OK on the next window. The following will appear: "Select the device driver you want to install for this hardware." You will also notice a warning that the driver is not digitally signed. Click NEXT. A Windows XP warning will appear: "This driver is not digitally signed." Click CONTINUE ANYWAY.

11. The wizard will install the FTDIBUS.INF driver. When finished, the following message will appear: "The wizard has finished installing the software for ID TECH - USB High Speed Serial Converter.

12. Click FINISH.

13. A "Found new hardware wizard" window should again appear on the screen.

14. As before, select "Install from a list or specific location" and click NEXT. The host should display another window: "Please choose your search and installation options"

15. Select "Don't search, I will choose the driver to install" and click NEXT. You will be asked to select the hardware type. Select "Show all devices" and click NEXT.

16. You will be asked to "select the device driver you want to install for this hardware."  Select SHOW ALL DEVICES and HAVE DISK and then click NEXT.

17. The INSTALL FROM DISK window will appear. Click BROWSE.

18. The LOCATE FILE window will appear. Find C:\ID TECH USB Serial Driver V3.0. Select FTDIPORT.INF and click OPEN.

20. Click OK on the next window. The following will appear: "Select the device driver you want to install for this hardware." You will also notice a warning that the driver is not digitally signed. Click NEXT. A Windows XP warning will appear: "This driver is not digitally signed." Click CONTINUE ANYWAY.

21. The wizard will install the FTDIPORT.INF driver. When finished, the following message will appear: "The wizard has finished installing the software for ID TECH - USB High Speed Serial Port.

22. Click FINISH. Both drivers are now installed, and the USB/RS-232 device should be fully functional.

For Type A ID TECH USB/RS-232 devices, it will be necessary to reinstall the driver for each unit connected to the host. For Type B ID TECH USB/RS-232 devices, it will only be necessary to install the driver once for each PC and USB port.

Checking the Installation and Modifying the COM Port Number

1. Go to START/CONTROL PANEL/SYSTEM/HARDWARE/DEVICE MANAGER/

PORTS (COM & LPT). Find "ID TECH USB Serial Port (COMX)" (where "X" can be any COM port number.) If you cannot find it, please go to "Troubleshooting."

2. Double click on "ID TECH USB Serial Port (COMX)"

3. Select PORT SETTINGS/ADVANCED/COM PORT NUMBER and change it to a valid number (usually COM3 or COM4).

4. Click OK on all open windows to close them.

For Type A ID TECH USB/RS-232 devices, each device will be assigned to a unique virtual com port number. For Type B ID TECH USB/RS-232 devices, each USB port will be assigned a unique virtual com port number.

Telling Whether the Installed Device is Type A or Type B

1. Go to START/CONTROL PANEL/SYSTEM/HARDWARE/DEVICE MANAGER.

2. Under PORTS (COM & LPT), a Type B ID TECH device will have a dash (-) between the words TECH and USB. An Type A ID TECH device will not have a dash between the words TECH and USB.

Older Device: 		“ID TECH USB Serial Port (COMX)”

Newer Device: 	“ID TECH-USB Serial Port (COMX)”

Under UNIVERSAL SERIAL BUS CONTROLLERS, a Type B ID TECH device will have a dash (-) between the words TECH and USB. An Type A ID TECH device will not have a dash between the words TECH and USB.

Older Device: 		“ID TECH USB Serial Converter)”

Newer Device: 	“ID TECH-USB Serial Converter)”

Troubleshooting

For most PCs, the installation of this new driver will be trouble-free. Occasionally, however, it may be necessary to update the driver or remove it entirely and re-install it several times in order to make it work.

If the driver did not install properly, try updating it first. If this does not fix the problem, remove the driver entirely.

Updating the Driver

1. With the reader plugged in, go to START/CONTROL PANEL/SYSTEM/HARDWARE/DEVICE MANAGER/PORTS (COM & LPT) and/or UNIVERSAL SERIAL BUS CONTROLLERS (as appropriate).

2. Locate any items like “Unknown USB Device” or “(ID TECH) USB Serial Converter” or “(ID TECH)USB Serial Port” that are marked with a yellow “!” These need to be updated.

3. Right-click on the item and select PROPERTIES/DRIVER and click UPDATE DRIVER.

4. The HARDWARE UPDATE WIZARD window will open. Select "Install from a list or specific location." Click NEXT.

5. The host should display another window: "Please choose your search and installation options." Select "Don't search, I will choose the driver to install" and click NEXT. You will be asked to select the hardware type. Select "Show all devices" and click NEXT.

7. You will be asked to "select the device driver you want to install for this hardware." Select HAVE DISK and then click NEXT.

8. The INSTALL FROM DISK window will appear. Click BROWSE.

9. The LOCATE FILE window will appear. Find C:\ID TECH USB Serial Driver V3.0. Select FTDIBUS.INF or FTDIPORT.INF (as appropriate) and click OPEN.

10. Click OK on the next window. You will notice a warning that the driver is not digitally signed. Click NEXT. A Windows XP warning will appear: "This driver is not digitally signed." Click CONTINUE ANYWAY.

11. The wizard will update the driver. When finished, the message "The wizard has finished installing... " will appear.

12. Click FINISH.

13. Repeat the above steps if both drivers must be updated.

Removing the Driver

1. With the reader plugged in, go to START/CONTROL PANEL/SYSTEM/HARDWARE/DEVICE MANAGER/PORTS (COM & LPT) and/or UNIVERSAL SERIAL BUS CONTROLLERS.

2. Locate any items like “Unknown USB Device” or “(ID TECH) USB Serial Converter” or “(ID TECH)USB Serial Port” marked with a yellow “!”

3. Right-click on the item and select the UNINSTALL option to remove the driver.

4. The CONFIRM DEVICE REMOVAL window will open. Click OK.

5. After the driver has been removed, unplug and then plug the reader in again.

6. A "Found new hardware wizard" window should appear on the screen. Install the driver again.